Mehran University of Engineering & Technology Jamshoro

Subject:     Electronic Instrumentation Systems

Assignment

Finger Gesture Controlled Servo System

Using

Machine Learning

Submitted by

DANIYAL 22ES077

1. Introduction

Machine Learning and intelligent automation are becoming important in modern electronic instrumentation systems. Traditional control systems use fixed rules and manual calibration, whereas Machine Learning enables systems to learn from data and make intelligent decisions automatically.

This project presents a Finger Gesture Controlled Servo System using Computer Vision and Machine Learning. The system detects finger movement through a webcam and controls a servo motor based on the distance between the thumb and index finger.

The system uses:

- OpenCV for image processing

- MediaPipe for hand tracking

- ESP32 for servo control

- Machine Learning models for prediction and intelligent decision making

The project demonstrates real-time automation and intelligent control using sensor data and Machine Learning algorithms.

2. Objectives

The objectives of this project are:

- To detect hand gestures using a webcam

- To calculate finger distance using image processing

- To control servo motor angle based on finger movement

- To apply Machine Learning for intelligent prediction

- To evaluate system performance using graphs and accuracy metrics

3. Instrumentation System Description

3.1 Components Used

| Component | Purpose |
| --- | --- |
| Webcam | Captures hand image |
| MediaPipe | Detects hand landmarks |
| OpenCV | Image processing |
| Arduino / ESP32 | Servo motor control |
| Servo Motor | Rotational movement |

3.2 Working Principle

The webcam captures the user’s hand image in real time. MediaPipe detects the landmarks of the hand and extracts the coordinates of the thumb tip and index finger tip.

The Euclidean distance between these two fingers is calculated. This distance is converted into a servo motor angle. The servo motor rotates clockwise or anticlockwise according to the predicted angle.

Machine Learning is used to:

- Predict servo angle

- Predict movement direction

Figure 1: Hardware Setup

3.3 System Flow Diagram

Figure 3: Block Diagram

4. Data Collection Method

The dataset was generated using finger distance and corresponding servo motor angle values.

The distance between thumb and index finger was recorded and mapped to servo motor angle.

4.1 Parameters Recorded

| Parameter | Description |
| --- | --- |
| Distance | Finger distance |
| Servo_Angle | Servo rotation angle |
| Direction | Clockwise / Anticlockwise / Center |

4.2 Dataset Size

- Total Samples: 600

Figure 4: Dataset Screenshot

5. Data Preprocessing

The dataset was processed using Python in Google Colab.

5.1 Preprocessing Steps

- Loaded dataset using pandas

- Checked missing values

- Encoded direction labels

- Visualized dataset using graphs

5.2 Direction Encoding

| Direction | Encoded Value |
| --- | --- |
| ANTICLOCKWISE | 0 |
| CENTER | 1 |
| CLOCKWISE | 2 |

Figure 5: Distance vs Servo Angle

Explanation

This graph shows the relationship between finger distance and servo motor angle.

6. Machine Learning Method

Two Machine Learning models were used in this project.

6.1 Linear Regression

Linear Regression was used to predict the servo motor angle based on finger distance.

Inputs

- Finger Distance

Output

- Servo Angle

6.2 Decision Tree Classifier

Decision Tree was used to predict servo movement direction.

Inputs

- Distance

- Servo Angle

Output

- Clockwise

- Anticlockwise

- Center

6.3 Training Process

The dataset was divided into:

- 80% Training Data

- 20% Testing Data

The models were trained using Scikit-learn library.

7. Results and Analysis

The system performance was evaluated using prediction graphs and accuracy metrics.

7.1 Regression Results

The Linear Regression model successfully predicted servo angle based on finger distance.

Figure 6: Actual vs Predicted Servo Angle

Explanation

The graph compares actual servo angle values with predicted values.

7.2 Error Analysis

The following metrics were used:

- Mean Absolute Error (MAE)

- Root Mean Square Error (RMSE)

These metrics show prediction accuracy of the regression model.

7.3 Classification Results

The Decision Tree model successfully predicted movement direction.

Figure 7: Confusion Matrix

Explanation

The confusion matrix shows correct and incorrect predictions.

Figure 8: Classification Accuracy Graph

Explanation

The graph shows classification accuracy of the Decision Tree model.

8. Intelligent System Feature

The system demonstrates intelligent automation by predicting:

- Servo angle

- Servo movement direction

The system automatically responds to finger movement without manual control.

Figure 9: Output Prediction

9. System Integration

The project integrates:

- Computer Vision

- Electronic Instrumentation

- Machine Learning

- Real-Time Control

Traditional systems use fixed conditions, while this ML-based system learns from data and improves decision making.

10. Advantages of the System

- Real-time control

- Contactless interaction

- Intelligent automation

- Easy human-machine interaction

- Accurate servo movement prediction

11. Applications

- Robotics

- Smart automation

- Gesture-controlled systems

- Industrial control

- Assistive technology

12. Conclusion

A Finger Gesture Controlled Servo System was successfully developed using Machine Learning and Computer Vision.

The system detects finger movement, predicts servo angle, and controls servo direction intelligently. Machine Learning improves the system’s prediction capability and automation performance.

The project demonstrates the integration of instrumentation systems with intelligent data-driven techniques.

14. Source Code

14.1 Python Code

# ====================================================

# 1. IMPORT LIBRARIES

# ====================================================

import pandas as pd

import matplotlib.pyplot as plt

import seaborn as sns

# ====================================================

# 2. LOAD DATASET

# ====================================================

data = pd.read_csv("finger_servo_dataset.csv")

print("First 5 Rows:\n")

print(data.head())

# ====================================================

# 3. CHECK DATASET INFORMATION

# ====================================================

print("\nDataset Information:\n")

print(data.info())

print("\nMissing Values:\n")

print(data.isnull().sum())

# ====================================================

# 4. DATA VISUALIZATION

# ====================================================

# Distance vs Servo Angle

plt.figure()

plt.scatter(data['Distance'], data['Servo_Angle'])

plt.xlabel("Finger Distance")

plt.ylabel("Servo Angle")

plt.title("Distance vs Servo Angle")

plt.show()

# ====================================================

# 5. ENCODE DIRECTION LABELS

# ====================================================

data['Direction_Encoded'] = data['Direction'].map({

'ANTICLOCKWISE': 0,

'CENTER': 1,

'CLOCKWISE': 2

})

print("\nEncoded Dataset:\n")

print(data.head())

# ====================================================

# 6. MACHINE LEARNING - REGRESSION

# Predict Servo Angle

# ====================================================

from sklearn.model_selection import train_test_split

from sklearn.linear_model import LinearRegression

# Input and Output

X_reg = data[['Distance']]

y_reg = data['Servo_Angle']

# Split Dataset

X_train_reg, X_test_reg, y_train_reg, y_test_reg = train_test_split(

X_reg, y_reg, test_size=0.2

)

# Train Model

reg_model = LinearRegression()

reg_model.fit(X_train_reg, y_train_reg)

print("\nLinear Regression Model Trained Successfully!")

# Prediction

y_pred_reg = reg_model.predict(X_test_reg)

# ====================================================

# 7. REGRESSION PERFORMANCE

# ====================================================

from sklearn.metrics import mean_absolute_error, mean_squared_error

import numpy as np

mae = mean_absolute_error(y_test_reg, y_pred_reg)

rmse = np.sqrt(mean_squared_error(y_test_reg, y_pred_reg))

print("\nMean Absolute Error:", mae)

print("Root Mean Square Error:", rmse)

# ====================================================

# 8. REGRESSION GRAPH

# ====================================================

plt.figure()

plt.scatter(X_test_reg, y_test_reg, label="Actual")

plt.scatter(X_test_reg, y_pred_reg, label="Predicted")

plt.xlabel("Finger Distance")

plt.ylabel("Servo Angle")

plt.title("Actual vs Predicted Servo Angle")

plt.legend()

plt.show()

# ====================================================

# 9. MACHINE LEARNING - CLASSIFICATION

# Predict Servo Direction

# ====================================================

from sklearn.tree import DecisionTreeClassifier

# Inputs and Outputs

X_cls = data[['Distance', 'Servo_Angle']]

y_cls = data['Direction_Encoded']

# Split Dataset

X_train_cls, X_test_cls, y_train_cls, y_test_cls = train_test_split(

X_cls, y_cls, test_size=0.2

)

# Train Classifier

cls_model = DecisionTreeClassifier()

cls_model.fit(X_train_cls, y_train_cls)

print("\nDecision Tree Classifier Trained Successfully!")

# Prediction

y_pred_cls = cls_model.predict(X_test_cls)

# ====================================================

# 10. CLASSIFICATION ACCURACY

# ====================================================

from sklearn.metrics import accuracy_score

accuracy = accuracy_score(y_test_cls, y_pred_cls)

print("\nClassification Accuracy:", accuracy)

# ====================================================

# 11. CONFUSION MATRIX

# ====================================================

from sklearn.metrics import confusion_matrix

cm = confusion_matrix(y_test_cls, y_pred_cls)

plt.figure()

sns.heatmap(

cm,

annot=True,

fmt='d',

xticklabels=['ANTICLOCKWISE','CENTER','CLOCKWISE'],

yticklabels=['ANTICLOCKWISE','CENTER','CLOCKWISE']

)

plt.xlabel("Predicted")

plt.ylabel("Actual")

plt.title("Confusion Matrix")

plt.show()

# ====================================================

# 12. ACCURACY GRAPH

# ====================================================

plt.figure()

plt.bar(["Decision Tree"], [accuracy])

plt.ylim(0,1)

plt.ylabel("Accuracy")

plt.title("Classification Accuracy")

plt.show()

# ====================================================

# 13. TEST WITH NEW INPUT

# Intelligent Feature

# ====================================================

sample = pd.DataFrame([[120]], columns=['Distance'])

predicted_angle = reg_model.predict(sample)

print("\nPredicted Servo Angle:", predicted_angle[0])

# Direction Prediction

sample2 = pd.DataFrame(

[[120, predicted_angle[0]]],

columns=['Distance', 'Servo_Angle']

)

direction_prediction = cls_model.predict(sample2)

direction_map = {

0: "ANTICLOCKWISE",

1: "CENTER",

2: "CLOCKWISE"

}

print("Predicted Direction:",

direction_map[direction_prediction[0]])

ESP32 Code

import cv2

import mediapipe as mp

import serial

import time

import math

# CHANGE YOUR COM PORT

esp = serial.Serial('COM3', 115200)

time.sleep(2)

mp_hands = mp.solutions.hands

hands = mp_hands.Hands(max_num_hands=1)

draw = mp.solutions.drawing_utils

cap = cv2.VideoCapture(0)

def distance(x1, y1, x2, y2):

return math.sqrt((x2-x1)**2 + (y2-y1)**2)

while True:

success, img = cap.read()

img = cv2.flip(img, 1)

rgb = cv2.cvtColor(img, cv2.COLOR_BGR2RGB)

result = hands.process(rgb)

if result.multi_hand_landmarks:

for hand in result.multi_hand_landmarks:

draw.draw_landmarks(img, hand)

h, w, c = img.shape

# Thumb tip (4) and Index tip (8)

x1 = int(hand.landmark[4].x * w)

y1 = int(hand.landmark[4].y * h)

x2 = int(hand.landmark[8].x * w)

y2 = int(hand.landmark[8].y * h)

dist = distance(x1, y1, x2, y2)

# Convert distance to angle

angle = int(dist / 2)

# limit 0–180

if angle < 0:

angle = 0

if angle > 180:

angle = 180

# send to ESP32

esp.write(f"{angle}\n".encode())

print("Angle:", angle)

cv2.imshow("Finger Control Servo", img)

if cv2.waitKey(1) & 0xFF == ord('q'):

break

14. References

- Arduino IDE Documentation

- Scikit-learn Documentation

- OpenCV Documentation

- MediaPipe Documentation